| * Copyright (c) 2021, Leopard Imaging Inc. All Rights Reserved. |  |
| --- | --- |
| Overview |  |
| This driver is for LI-AR0234CS-STEREO-GMSL2 V1.0 camera kit with Nvidia Jetson AGX/Orin Xavier Developer kit. This driver supports two AR0234CS-STEREO-GMSL2 cameras. This driver supports 1920x1200@30fps,1920x1200@60fps，960x600@120fps This driver is based on R35.2 GA. |  |
| Download link |  |
| Platform | Camera |
| Nvidia Jetson AGX Xavier/Orin Developer kit | 2 ~ LI-AR0234CS-STEREO-GMSL2_V1.0 |
| Cable | Adapter/Carrier Board |
| 1 x FAK-SMZSMZ | 1 x NVIDIA max96712 adapt board(E3653-a03). 1 x 19VDC power supply |

| Revision |  | SVN version | Release Date | Author | Tested by |  |
| --- | --- | --- | --- | --- | --- | --- |
| 20210414 |  |  | 04/14/2021 | Xingxing Gu |  |  |
| Updates |  |  |  |  |  |  |
| Revision | Description |  |  |  |  | Release Date |
| 20210414 | First Release based on R32.5 |  |  |  |  | 20210414 |
| 20210511 | Support NVS imu driver |  |  |  |  | 20210511 |
| 20210705 | Support autoContect Switch |  |  |  |  | 20210705 |
| 20210812 | Support 32.6 |  |  |  |  | 20210812 |
| 20210918 | Support NVS |  |  |  |  | 20210918 |
| 20220301 | Support 60fps |  |  |  |  | 20220301 |
| 20220310 | Support 32.7 JP4.6.1 |  |  |  |  | 20220310 |
| 20220316 | Fix the one camera issue |  |  |  |  | 20220316 |
| 20220318 | Fix flicker issue |  |  |  |  | 20220318 |
| 20220415 | Support JP50 |  |  |  |  | 20220415 |
| 20220526 | Support JP50.1 add 960x600@120fps |  |  |  |  | 20220526 |
| 20220530 | Support Orin |  |  |  |  | 20220530 |
| 20220715 | Support IMU |  |  |  |  | 20220715 |
| 20220720 | Support IMU timestamp |  |  |  |  | 20220720 |
| 20220822 | Support 35.1 |  |  |  |  | 20220822 |
| 20220922 | Fix the ethernet issue when install the ko |  |  |  |  | 20220922 |
| 20220927 | Fix the imu issue on Orin |  |  |  |  | 20220927 |
| 20221008 | Fix the hawk 60fps issue. |  |  |  |  | 20221008 |
| 20221011 | Fix the Xavier issue |  |  |  |  | 20221011 |
| 20221028 | Remove second hawk imu interrupt pin |  |  |  |  | 20221028 |
| 20230116 | Support R35.2 EA |  |  |  |  | 20230116 |
| 20230130 | Support R35.2 GA |  |  |  |  | 20230130 |
| Known bugs |  |  |  |  |  |  |
| - If you only have one hawk camera, you can only connect port 1 |  |  |  |  |  |  |

Setup Procedure 1/2

Driver installation: 1. Download the R35.1 OS Image (from link below) to your Ubuntu OS on Intel x64 Host PC (we are using Ubuntu 18.04, virtual machine is fine) and follow the l4t_quick_start_guide to install the Jetpack to Xavier. 3. Copy the tegra194-p2888-0001-p2822-0000.dtb or tegra234-p3701-0000-p3737-0000.dtb to the /boot/dtb/kernel_ tegra194-p2888-0001-p2822-0000.dtb or /boot/dtb/kernel_tegra234-p3701-0000-p3737-0000.dtb in your Xavier/Orin platform.

Setup Procedure 2/2

5. After boot up Xavier/Orin, copy “Image” to /boot on Xavier. 6. Plug in 19V power supply to Xavier/Orin kit. 7. insmod max96712.ko, ar0234.ko bmi088.ko orderly. 8. open a terminal and do below command. You will get live video output. $ nvgstcapture 8. Use Ctrl+C to close the video and copy camera_overrides.isp to /var/nvidia/nvcam/settings on Xavier/Orin and do below two command. $ sudo chmod 664 /var/nvidia/nvcam/settings/camera_overrides.isp $ sudo chown root:root /var/nvidia/nvcam/settings/camera_overrides.isp

Run Camera

1. Argus software Download the Multimedia package from link below and copy it to Xavier/Orin. Open a terminal, do sudo apt-get update sudo apt-get install cmake libgtk-3-dev libjpeg-dev libgles2-mesa-dev libgstreamer1.0-dev Uncompress the tgz package, tar zxvf Multimedia_JXAV_R35.1.tgz Under tegra_multimedia_api/argus/cmake, do cmake .. make sudo make install Do "argus_camera --device=0” to get the video. 2. Gstreamer gst-launch-1.0 nvarguscamerasrc sensor-id=0 ! 'video/x-raw(memory:NVMM), width=(int)1920, height=(int)1200, framerate=30/1' ! nvvidconv flip-method=0 ! 'video/x-raw, format=(string)I420' ! xvimagesink -e 3. v4l2-ctl capture raw v4l2-ctl -V --set-fmt-video=width=1920,height=1200,pixelformat=RG10 --set-ctrl bypass_mode=0 --stream-mmap --stream-count=1 --stream-to=ar0234.raw -d /dev/video0 Please use below commands to install v4l2. sudo apt-get update sudo apt-get install v4l-utils

Note 1/2

1. If you would like to install the Jetpack 5.0 but don’t want to re-flash the whole OS image, you can uncheck the Jetson OS and install the Jetson SDK components only.

Note 2/2

4. Compile the driver TBD

IMU support

- How to access imu data in driver Acc: 1.cd /sys/devices/platform/3180000.i2c/i2c-2/i2c-30/30-0069/iio:device0 cd scan_elements echo 1 >in_accel_x_en echo 1 > in_accel_y_en echo 1 > in_accel_z_en echo 1 > in_timestamp_en cd ../buffer echo 1 > enable cd .. cat in_accel_x_raw cat in_accel_y_raw cat in_accel_y_raw geo : 1.cd /sys/devices/platform/3180000.i2c/i2c-2/i2c-30/30-0069/iio:device1 cd scan_elements echo 1 >in_anglvel_x_en echo 1 >in_anglvel_y_en echo 1 > in_anglvel_z_en echo 1 > in_timestamp_en cd ../buffer echo 1 > enable cd .. cat in_ anglvel_x_raw cat in_ anglvel_y_raw cat in_ anglvel_y_raw